TINY KERNEL

Standalone Training + Reproduction Manual

v0.1 -> v0.7 | QEMU ARM64 / AArch64 | Boot -> GDB -> MMIO -> Exceptions -> Timer/GIC -> Memory -> One-Task Execution

Reproducibility target: a new engineer should be able to start with a fresh Linux machine, follow this document only, and reproduce every completed milestone through v0.7 without the original chat.

Document purpose: This is the single consolidated source of truth for the Tiny Kernel work completed through v0.7. It preserves useful debugging history while remaining a standalone lab manual. v0.7 is now closed; the next milestone has not started.

| Target | AArch64 / ARMv8-A |
| --- | --- |
| Virtual platform | QEMU virt, Cortex-A53 model, 128 MB RAM |
| Host tooling | Linux + aarch64-linux-gnu toolchain + QEMU + gdb-multiarch |
| Project path | ~/project_Tiny_kernel/tiny-kernel |
| Current completed milestone | v0.7 One-task execution + v0.6 memory/timer/GIC regression |

# 1. What "standalone" means for this document

The document is successful only when it works as an executable training manual, not merely as a record of what happened. A new engineer should not need hidden context to decide which address, file, command, or correction is authoritative.

## 1.1 Reproducibility contract

Every completed stage has a stated goal, exact build/run commands, final expected result, acceptance criteria, and known failure modes.

The v0.5 base plus the final v0.6 allocator/linker/build snapshot is included, so the completed kernel can be reproduced from the document without the original chat.

Historical mistakes are retained only where they teach a debugging method or explain why the final design looks the way it does.

Any address shown as final is explicitly labelled as the verified QEMU virt value for this lab.

## 1.2 Golden engineering loop

implement
|
build
|
inspect ELF / image
|
boot
|
attach GDB
|
observe exact instruction / register / MMIO state
|
reproduce fault
|
fix one cause
|
rebuild + rerun
|
record acceptance result

# 2. Master roadmap and completion gates

| Milestone | What must be proven | Status |
| --- | --- | --- |
| v0.1 | Boot raw AArch64 image, set stack, call C, print over UART | PASS |
| v0.2 | Understand QEMU hand-off and prove _start -> kernel_main with GDB | PASS |
| v0.3 | Map source -> register -> UART MMIO at instruction level | PASS |
| v0.4 | Install exception vectors and capture EL1 fault state | PASS |
| v0.5 | Generate timer IRQ, acknowledge INTID 27, reload timer, return with ERET | PASS |
| v0.6 | Minimal memory/page-frame allocator, free/reuse, negative tests, exhaustion, GDB bitmap proof | PASS |
| v0.7 | One-task execution: task lifecycle, dedicated stack, task entry/return, stack reuse, negative tests, GDB stack proof | PASS |
| v0.8 | Peripheral bus driver | PENDING |
| v0.9 | IoT sensor driver | PENDING |
| v1.0+ | Hardening + Raspberry Pi port | PENDING |

## 2.1 Rule before v0.6 (historical gate)

v0.6 was started only after the v0.5 acceptance gate was PASS on the same machine. v0.6 was then completed and frozen before v0.7. v0.7 is now complete and frozen before the next milestone.

# 3. Environment and prerequisites

sudo apt update
sudo apt install -y gcc-aarch64-linux-gnu binutils-aarch64-linux-gnu qemu-system-arm

aarch64-linux-gnu-gcc --version
qemu-system-aarch64 --version
gdb-multiarch --version

The working debugger used during the lab was gdb-multiarch. A separate aarch64-linux-gnu-gdb executable is not required for this setup.

## 3.1 Project tree

mkdir -p ~/project_Tiny_kernel/tiny-kernel/{boot,kernel,drivers,include}
cd ~/project_Tiny_kernel/tiny-kernel
find .

.
./boot
./kernel
./drivers
./include

Note: drivers/ remains reserved for later milestones. include/ is now used by v0.7 for task.h. The v0.7 task subsystem lives in include/task.h, kernel/task.c, and kernel/task_switch.S.

# 4. v0.1 - Boot + UART

Goal: produce a freestanding AArch64 raw image that QEMU can enter, establish a stack, call kernel_main(), and write a known string through the QEMU virt PL011 UART.

## 4.1 Final v0.1 source snapshot

[ASM]
// boot/start.S
.section .text.boot
.global _start

_start:
    ldr x0, =stack_top
    mov sp, x0

    bl kernel_main

1:
    wfe
    b 1b

[C]
// kernel/main.c
#define UART_BASE 0x09000000UL
#define UART_DR   0x00
#define UART_FR   0x18

static inline void uart_putc(char c)
{
    volatile unsigned int *uart =
        (volatile unsigned int *)UART_BASE;

    while (uart[UART_FR / 4] & (1 << 5))
        ;

    uart[UART_DR / 4] = c;
}

static void uart_puts(const char *s)
{
    while (*s)
        uart_putc(*s++);
}

void kernel_main(void)
{
    uart_puts("TINY KERNEL STARTED\r\n");

    while (1)
        ;
}

[LD]
/* linker.ld */
ENTRY(_start)

SECTIONS
{
    . = 0x40080000;

    .text : ALIGN(16)
    {
        KEEP(*(.text.boot))
        *(.text*)
    }

    .rodata : ALIGN(16)
    {
        *(.rodata*)
    }

    .data : ALIGN(16)
    {
        *(.data*)
    }

    .bss (NOLOAD) : ALIGN(16)
    {
        __bss_start = .;
        *(.bss*)
        *(COMMON)
        __bss_end = .;

        . = ALIGN(16);
        . += 0x10000;

        stack_top = .;
    }

    /DISCARD/ :
    {
        *(.comment*)
        *(.note*)
        *(.eh_frame*)
        *(.gnu.build.attributes*)
    }
}

[MAKE]
# Makefile
CROSS_COMPILE = aarch64-linux-gnu-

CC      = $(CROSS_COMPILE)gcc
LD      = $(CROSS_COMPILE)ld
OBJCOPY = $(CROSS_COMPILE)objcopy

CFLAGS = -ffreestanding \
         -fno-stack-protector \
         -fno-pie \
         -fno-builtin \
         -Wall \
         -Wextra \
         -O0 \
         -g

LDFLAGS = -T linker.ld \
          --nostdlib \
          --static \
          --gc-sections \
          --build-id=none \
          -z max-page-size=0x1000

OBJS = boot/start.o kernel/main.o

all: kernel.elf kernel.bin

boot/start.o: boot/start.S
	$(CC) $(CFLAGS) -c $< -o $@

kernel/main.o: kernel/main.c
	$(CC) $(CFLAGS) -c $< -o $@

kernel.elf: $(OBJS) linker.ld
	$(LD) $(LDFLAGS) $(OBJS) -o $@

kernel.bin: kernel.elf
	$(OBJCOPY) -O binary $< $@

clean:
	rm -f $(OBJS) kernel.elf kernel.bin

.PHONY: all clean

## 4.2 Build and verify

make clean
make
aarch64-linux-gnu-readelf -h kernel.elf | grep "Entry point"
aarch64-linux-gnu-readelf -s kernel.elf | grep -E "_start|kernel_main|stack_top"
od -Ax -tx1 -N32 kernel.bin

Final verified v0.1 runtime layout uses .text at 0x40080000. The first attempt used 0x40000000; GDB analysis in v0.2 showed that address contained QEMU's ARM64 boot stub, so 0x40080000 became the reproducible kernel entry.

## 4.3 First known failure: GNU build-id

The first raw binary contained a .note.gnu.build-id section before the startup code. QEMU therefore did not see the expected instruction stream at the image start. The final build avoids this by linking directly with aarch64-linux-gnu-ld, passing --build-id=none, and discarding .note* sections in linker.ld.

Observed bad raw beginning:
04 00 00 00 14 00 00 00 03 00 00 00 47 4e 55 ...

Final check:
od -Ax -tx1 -N32 kernel.bin

## 4.4 Boot

qemu-system-aarch64 \
 -M virt \
 -cpu cortex-a53 \
 -m 128M \
 -nographic \
 -monitor none \
 -kernel kernel.bin

Expected:
TINY KERNEL STARTED

# 5. v0.2 - QEMU hand-off + GDB CPU tracing

Goal: prove where QEMU starts execution, distinguish QEMU boot code from our own _start, and trace the transition into C.

## 5.1 Start QEMU paused

qemu-system-aarch64 \
 -M virt \
 -cpu cortex-a53 \
 -m 128M \
 -nographic \
 -monitor none \
 -kernel "$PWD/kernel.bin" \
 -S \
 -gdb tcp::1234

## 5.2 Connect GDB

gdb-multiarch ./kernel.elf
set architecture aarch64
target remote :1234

Note: Use target remote :1234. The earlier typo set target :1234 is invalid.

## 5.3 QEMU boot stub discovery

x/8i 0x40000000
x/wx 0x40000020
si
info registers x4
break *$x4
si
p/x $pc
x/5i $pc

Observed QEMU boot stub:
0x40000000: ldr x0, 0x40000018
0x40000004: mov x1, xzr
0x40000008: mov x2, xzr
0x4000000c: mov x3, xzr
0x40000010: ldr x4, 0x40000020
0x40000014: br x4

Observed:
0x40000018 = 0x44000000  (DTB / boot-data address)
0x40000020 = 0x40080000  (our kernel entry)

This explained two confusing values from the first debugging session: SP=0 was normal before our own stack setup, and x0=0x44000000 was QEMU's boot-data/DTB argument rather than stack_top.

## 5.4 Prove _start -> stack -> kernel_main

si
p/x $pc
info registers x0
si
p/x $pc
info registers sp
si
p/x $pc
info registers x30

Verified pattern:
PC = 0x40080004, x0 = stack_top
PC = 0x40080008, SP = stack_top
PC = kernel_main, x30 = return address 0x4008000c

## 5.5 Correct breakpoint syntax

To break at the address stored in x4, use break *$x4.

To execute one instruction, use si. Do not use continue when you intend to inspect every instruction.

Always inspect p/x $pc before interpreting the other registers.

# 6. v0.3 - Source -> register -> UART MMIO

Goal: add DWARF debug information and trace one complete character from C source through AArch64 registers and into the UART MMIO write.

## 6.1 Build with -g

CFLAGS = -ffreestanding \
         -fno-stack-protector \
         -fno-pie \
         -fno-builtin \
         -Wall \
         -Wextra \
         -O0 \
         -g

After this change, GDB can map instruction addresses to exact C source lines.

## 6.2 Split UART into its own source file

[C]
// kernel/uart.c
#define UART_BASE 0x09000000UL
#define UART_DR   0x00
#define UART_FR   0x18

static inline void uart_putc(char c)
{
    volatile unsigned int *uart =
        (volatile unsigned int *)UART_BASE;

    while (uart[UART_FR / 4] & (1 << 5))
        ;

    uart[UART_DR / 4] = c;
}

void uart_puts(const char *s)
{
    while (*s)
        uart_putc(*s++);
}

[C]
// kernel/main.c
extern void uart_puts(const char *s);

void kernel_main(void)
{
    uart_puts("TINY KERNEL STARTED\r\n");

    while (1)
        ;
}

## 6.3 GDB source trace

break kernel_main
continue
list
info line
step
info registers x0
x/s $x0
next
step
info registers x0
p/c (char)$x0

Observed:
uart_puts(s=0x400800c0 "TINY KERNEL STARTED\r\n")
x0 = 0x400800c0

First character:
uart_putc(c=84 'T')
x0 = 0x54
0x54 = ASCII 'T'

## 6.4 UART MMIO proof

x/12i $pc
x/wx 0x09000018

Observed UART flag register:
0x09000018: 0x00000090

The driver checks bit 0x20.
0x90 & 0x20 = 0, so transmission is allowed.

Final store:
str w1, [x0]
with x0 = 0x09000000 (UART_DR)
and w1 holding the character value.

## 6.5 Value versus pointer rule

When x0 = 0x400800c0:
x/s $x0              # inspect memory as a string
x/c $x0              # inspect one byte at pointed memory

When x0 = 0x54 or 0x49:
p/c (char)$x0         # interpret the register value as a character

# 7. v0.4 - Exception handling

Goal: install an AArch64 vector table, program VBAR_EL1, deliberately execute BRK #0, enter a synchronous exception handler, capture exception state, and prove the fault path in QEMU/GDB.

## 7.1 Final v0.4 files

[ASM]
// boot/start.S
.section .text.boot
.global _start

_start:
    ldr x0, =stack_top
    mov sp, x0

    ldr x0, =vector_table
    msr vbar_el1, x0
    isb

    bl kernel_main

1:
    wfe
    b 1b

[ASM]
// kernel/exception.S
.section .text.vectors, "ax"
.balign 2048
.global vector_table

.macro VECTOR target
    b \target
    .space 124
.endm

vector_table:

    VECTOR sync_exception
    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception

    VECTOR sync_exception
    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception

    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception

    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception

.section .text.exception, "ax"

.global sync_exception
sync_exception:
    bl exception_report

1:
    wfe
    b 1b

.global default_exception
default_exception:
2:
    wfe
    b 2b

[C]
// kernel/exception.c
#include <stdint.h>

extern void uart_puts(const char *s);

volatile uint64_t captured_elr;
volatile uint64_t captured_esr;
volatile uint64_t captured_spsr;
volatile uint64_t captured_vbar;
volatile uint64_t captured_far;

void exception_report(void)
{
    asm volatile("mrs %0, elr_el1"  : "=r"(captured_elr));
    asm volatile("mrs %0, esr_el1"  : "=r"(captured_esr));
    asm volatile("mrs %0, spsr_el1" : "=r"(captured_spsr));
    asm volatile("mrs %0, vbar_el1" : "=r"(captured_vbar));
    asm volatile("mrs %0, far_el1"  : "=r"(captured_far));
    uart_puts("SYNC EXCEPTION\r\n");
}

[C]
// kernel/main.c for v0.4
extern void uart_puts(const char *s);

void kernel_main(void)
{
    uart_puts("BEFORE EXCEPTION\r\n");

    asm volatile("brk #0");

    while (1)
        ;
}

## 7.2 Required linker changes

[LD]
.text : ALIGN(16)
{
    KEEP(*(.text.boot))
    *(.text)
    *(.text.exception)
}

.text.vectors : ALIGN(2048)
{
    KEEP(*(.text.vectors))
}

## 7.3 Verify vector layout

aarch64-linux-gnu-readelf -s kernel.elf | grep -E "_start|vector_table|kernel_main|sync_exception|stack_top"

Verified v0.4 example:
_start       = 0x40080000
vector_table = 0x40080800
kernel_main  = 0x400800b8
sync_exception=0x400800f4
stack_top    = 0x40091030

## 7.4 Functional test

qemu-system-aarch64 \
 -M virt \
 -cpu cortex-a53 \
 -m 128M \
 -nographic \
 -monitor none \
 -kernel "$PWD/kernel.bin"

Expected / verified:
BEFORE EXCEPTION
SYNC EXCEPTION

## 7.5 Captured exception state

p/x captured_elr
p/x captured_esr
p/d ((captured_esr >> 26) & 0x3f)
p/x captured_spsr
p/x captured_vbar
p/x captured_far

Verified values:
ELR_EL1  = 0x400800cc
ESR_EL1  = 0xf2000000
EC       = 60
SPSR_EL1 = 0x600003c5
VBAR_EL1 = 0x40080800
FAR_EL1  = 0x0

A remote GDB session did not expose ELR_EL1 / ESR_EL1 / SPSR_EL1 / VBAR_EL1 as ordinary register names. The robust lab method was therefore to read them with MRS instructions inside the kernel and save them into volatile variables.

# 8. v0.5 - Timer + GIC interrupts

Goal: program the ARM generic timer, enable its interrupt, route the interrupt through the QEMU GICv2 model, enter the IRQ vector, acknowledge INTID 27, reload the timer, emit TICK, signal end-of-interrupt, and return with ERET.

## 8.1 Final v0.5 source snapshot

[ASM]
// boot/start.S
.section .text.boot
.global _start

_start:
    ldr x0, =stack_top
    mov sp, x0

    ldr x0, =vector_table
    msr vbar_el1, x0
    isb

    bl kernel_main

1:
    wfe
    b 1b

[C]
// kernel/uart.c
#define UART_BASE 0x09000000UL
#define UART_DR   0x00
#define UART_FR   0x18

static inline void uart_putc(char c)
{
    volatile unsigned int *uart =
        (volatile unsigned int *)UART_BASE;

    while (uart[UART_FR / 4] & (1 << 5))
        ;

    uart[UART_DR / 4] = c;
}

void uart_puts(const char *s)
{
    while (*s)
        uart_putc(*s++);
}

[C]
// kernel/gic.c
#include <stdint.h>

#define GICD_BASE          0x08000000UL
#define GICC_BASE          0x08010000UL

#define GICD_CTLR          0x000
#define GICD_IGROUPR0      0x080
#define GICD_ISENABLER0    0x100

#define GICC_CTLR          0x000
#define GICC_PMR           0x004
#define GICC_IAR           0x00C
#define GICC_AIAR         0x020
#define GICC_EOIR          0x010
#define GICC_AEOIR        0x024

#define TIMER_INTID        27

static inline void mmio_write32(uintptr_t addr, uint32_t value)
{
    *(volatile uint32_t *)addr = value;
}

static inline uint32_t mmio_read32(uintptr_t addr)
{
    return *(volatile uint32_t *)addr;
}

void gic_init(void)
{
    uint32_t group = mmio_read32(GICD_BASE + GICD_IGROUPR0);
    group |= (1U << TIMER_INTID);
    mmio_write32(GICD_BASE + GICD_IGROUPR0, group);

    mmio_write32(GICD_BASE + GICD_ISENABLER0,
                 (1U << TIMER_INTID));

    mmio_write32(GICC_BASE + GICC_PMR, 0xFF);
    mmio_write32(GICC_BASE + GICC_CTLR, 0x6);
    mmio_write32(GICD_BASE + GICD_CTLR, 0x2);
}

uint32_t gic_acknowledge(void)
{
    return mmio_read32(GICC_BASE + GICC_IAR);
}

void gic_end_interrupt(uint32_t iar)
{
    mmio_write32(GICC_BASE + GICC_EOIR, iar);
}

[C]
// kernel/timer.c
#include <stdint.h>

static uint64_t timer_period;

static inline uint64_t read_cntfrq(void)
{
    uint64_t value;
    asm volatile("mrs %0, cntfrq_el0" : "=r"(value));
    return value;
}

static inline void write_cntv_tval(uint64_t value)
{
    asm volatile("msr cntv_tval_el0, %0" :: "r"(value));
}

static inline void write_cntv_ctl(uint64_t value)
{
    asm volatile("msr cntv_ctl_el0, %0" :: "r"(value));
}

void timer_init(void)
{
    uint64_t frequency = read_cntfrq();
    timer_period = frequency / 2;
    write_cntv_tval(timer_period);
    write_cntv_ctl(1);
    asm volatile("isb");
}

void timer_reload(void)
{
    write_cntv_tval(timer_period);
    asm volatile("isb");
}

[C]
// kernel/exception.c
#include <stdint.h>

extern void uart_puts(const char *s);
extern uint32_t gic_acknowledge(void);
extern void gic_end_interrupt(uint32_t iar);
extern void timer_reload(void);

volatile uint64_t captured_elr;
volatile uint64_t captured_esr;
volatile uint64_t captured_spsr;
volatile uint64_t captured_vbar;
volatile uint64_t captured_far;
volatile uint64_t timer_ticks;

void exception_report(void)
{
    asm volatile("mrs %0, elr_el1"  : "=r"(captured_elr));
    asm volatile("mrs %0, esr_el1"  : "=r"(captured_esr));
    asm volatile("mrs %0, spsr_el1" : "=r"(captured_spsr));
    asm volatile("mrs %0, vbar_el1" : "=r"(captured_vbar));
    asm volatile("mrs %0, far_el1"  : "=r"(captured_far));
    uart_puts("SYNC EXCEPTION\r\n");
}

void irq_handler(void)
{
    uint32_t iar;
    uint32_t intid;

    iar = gic_acknowledge();
    intid = iar & 0x3FF;

    if (intid == 27) {
        timer_ticks++;
        timer_reload();
        uart_puts("TICK\r\n");
    }

    gic_end_interrupt(iar);
}

[ASM]
// kernel/exception.S
.section .text.vectors, "ax"
.balign 2048
.global vector_table

.macro VECTOR target
    b \target
    .space 124
.endm

vector_table:

    // Current EL with SP0
    VECTOR sync_exception
    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception

    // Current EL with SPx
    VECTOR sync_exception
    VECTOR irq_exception
    VECTOR default_exception
    VECTOR default_exception

    // Lower EL using AArch64
    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception

    // Lower EL using AArch32
    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception
    VECTOR default_exception

.section .text.exception, "ax"

.global sync_exception
sync_exception:
    bl exception_report

1:
    wfe
    b 1b

.global irq_exception
irq_exception:
    bl irq_handler
    eret

.global default_exception
default_exception:
2:
    wfe
    b 2b

[C]
// kernel/main.c
extern void uart_puts(const char *s);
extern void gic_init(void);
extern void timer_init(void);

void kernel_main(void)
{
    uart_puts("TIMER TEST\r\n");

    gic_init();
    timer_init();

    asm volatile("msr daifclr, #2");
    asm volatile("isb");

    while (1) {
        asm volatile("wfe");
    }
}

## 8.2 Final v0.5 linker script

[LD]
ENTRY(_start)

SECTIONS
{
    . = 0x40080000;

    .text : ALIGN(16)
    {
        KEEP(*(.text.boot))
        *(.text)
        *(.text.exception)
    }

    .text.vectors : ALIGN(2048)
    {
        KEEP(*(.text.vectors))
    }

    .rodata : ALIGN(16)
    {
        *(.rodata*)
    }

    .data : ALIGN(16)
    {
        *(.data*)
    }

    .bss (NOLOAD) : ALIGN(16)
    {
        __bss_start = .;
        *(.bss*)
        *(COMMON)
        __bss_end = .;

        . = ALIGN(16);
        . += 0x10000;

        stack_top = .;
    }

    /DISCARD/ :
    {
        *(.comment*)
        *(.note*)
        *(.eh_frame*)
        *(.gnu.build.attributes*)
    }
}

## 8.3 Final v0.5 Makefile

[MAKE]
CROSS_COMPILE = aarch64-linux-gnu-

CC      = $(CROSS_COMPILE)gcc
LD      = $(CROSS_COMPILE)ld
OBJCOPY = $(CROSS_COMPILE)objcopy

CFLAGS = -ffreestanding \
         -fno-stack-protector \
         -fno-pie \
         -fno-builtin \
         -Wall \
         -Wextra \
         -O0 \
         -g

LDFLAGS = -T linker.ld \
          --nostdlib \
          --static \
          --gc-sections \
          --build-id=none \
          -z max-page-size=0x1000

OBJS = boot/start.o \
       kernel/main.o \
       kernel/uart.o \
       kernel/exception.o \
       kernel/exception-vector.o \
       kernel/gic.o \
       kernel/timer.o

all: kernel.elf kernel.bin

boot/start.o: boot/start.S
	$(CC) $(CFLAGS) -c $< -o $@

kernel/main.o: kernel/main.c
	$(CC) $(CFLAGS) -c $< -o $@

kernel/uart.o: kernel/uart.c
	$(CC) $(CFLAGS) -c $< -o $@

kernel/exception.o: kernel/exception.c
	$(CC) $(CFLAGS) -c $< -o $@

kernel/exception-vector.o: kernel/exception.S
	$(CC) $(CFLAGS) -c $< -o $@

kernel/gic.o: kernel/gic.c
	$(CC) $(CFLAGS) -c $< -o $@

kernel/timer.o: kernel/timer.c
	$(CC) $(CFLAGS) -c $< -o $@

kernel.elf: $(OBJS) linker.ld
	$(LD) $(LDFLAGS) $(OBJS) -o $@

kernel.bin: kernel.elf
	$(OBJCOPY) -O binary $< $@

clean:
	rm -f $(OBJS) kernel.elf kernel.bin

.PHONY: all clean

## 8.4 Build and golden validation

cd ~/project_Tiny_kernel/tiny-kernel
make clean
make

aarch64-linux-gnu-readelf -h kernel.elf | grep "Entry point"
aarch64-linux-gnu-readelf -s kernel.elf | grep -E "_start|vector_table|kernel_main|irq_exception|stack_top"

qemu-system-aarch64 \
-M virt,gic-version=2 \
-cpu cortex-a53 \
-m 128M \
-nographic \
-monitor none \
-kernel kernel.bin

Verified golden output:
TIMER TEST
TICK
TICK
TICK
TICK
TICK
TICK
TICK
...

# 9. v0.5 debugging record - what failed and how it was diagnosed

This section is deliberately preserved because the debugging path teaches the exact method needed in BSP/bring-up work. It is not the recommended starting point when running the already-fixed v0.5 code.

## 9.1 Symptom: timer started, but no TICK

Initial QEMU result:
TIMER TEST

The first question was whether the timer interrupt was being generated at all. GDB breakpoints showed that timer_init() ran, the CPU reached irq_exception, and the CPU then reached irq_handler. Therefore the timer was not the primary failure.

## 9.2 Inspect GIC pending / acknowledge state

x/wx 0x08000200     # GICD_ISPENDR0
x/wx 0x08010014     # GICC_RPR
x/wx 0x08010018     # GICC_HPPIR
x/wx 0x0801000c     # GICC_IAR

Observed:
GICD_ISPENDR0 = 0x08000000
GICC_RPR       = 0x000000ff
GICC_HPPIR     = 0x000003fe
GICC_IAR       = 0x000003fe

0x08000000 in ISPENDR0 corresponds to bit 27, so INTID 27 was pending. The important conclusion was: timer generation and interrupt delivery to the CPU were working; the problem was in the GIC acknowledgement path.

## 9.3 Failed experiment: change IAR/CTL registers live from GDB

set {unsigned int}0x08010000 = 0x3
set {unsigned int}0x08000000 = 0x3
x/wx 0x08010000
x/wx 0x08000000

Readback remained:
GICC_CTLR = 0x2
GICD_CTLR = 0x2

Device control registers are not ordinary RAM. A write that is accepted, masked, security-filtered, or otherwise constrained by the emulated device does not behave like a RAM write/readback experiment. The result was treated as evidence, not as proof that the write should be blindly repeated.

## 9.4 Failed experiment: Group-1 aliased acknowledge registers

GICC_AIAR = 0x20
GICC_AEOIR = 0x24

In this QEMU configuration:
x/wx 0x08010020  -> 0x00000000
x/wx 0x08010024  -> 0x00000000

The code was temporarily switched to AIAR/AEOIR. That build succeeded, but the runtime still printed only TIMER TEST. The alias path was therefore not used in the final solution for this lab configuration.

## 9.5 Final fix: enable AckCtl and use normal IAR/EOIR

GICC_CTLR write changed:
0x2  -> 0x6

Final acknowledge:
return mmio_read32(GICC_BASE + GICC_IAR);

Final end-of-interrupt:
mmio_write32(GICC_BASE + GICC_EOIR, iar);

After rebuilding and running the same QEMU command, the timer produced repeated TICK output. This closed the loop from symptom -> pending interrupt evidence -> GIC acknowledgement diagnosis -> one controlled fix -> regression run.

# 10. Acceptance criteria - completed stages

## v0.1 PASS

Cross compiler builds kernel.elf and raw kernel.bin.

ELF entry point is 0x40080000.

QEMU prints TINY KERNEL STARTED.

CPU remains alive after the message.

## v0.2 PASS

QEMU boot stub at 0x40000000 is identified.

0x40000020 contains 0x40080000.

break *$x4 reaches our _start.

Stack is established from stack_top.

BL reaches kernel_main and x30 holds the return address.

## v0.3 PASS

GDB maps PC to C source with -g.

uart_puts receives a valid string pointer.

First character T is observed as 0x54.

UART_FR is accessed at 0x09000018.

UART_DR is accessed at 0x09000000.

The character store executes through str w1,[x0].

## v0.4 PASS

vector_table is 2048-byte aligned.

VBAR_EL1 is programmed to vector_table.

BRK #0 enters sync_exception.

ELR_EL1 matches the BRK instruction address.

ESR_EL1 EC decodes as 60.

SYNC EXCEPTION is printed.

## v0.5 PASS

QEMU is started with -M virt,gic-version=2.

Timer initialization executes.

CPU enters irq_exception.

irq_handler executes.

INTID 27 is acknowledged.

timer_reload() runs.

TICK is printed repeatedly.

EOI is written and ERET returns to the interrupted context.

# 11. Troubleshooting decision table

| Symptom | Likely evidence | Action |
| --- | --- | --- |
| QEMU cannot load kernel.bin | make failed or kernel.bin was removed by make clean | Fix the build first; rerun make and confirm kernel.bin exists. |
| No UART output | Wrong image / wrong entry / UART path not reached | Check readelf entry, od, then GDB at _start and kernel_main. |
| PC at 0x40000000 looks unrelated to start.S | QEMU boot stub is executing | Inspect x4 and 0x40000020; our raw kernel entry is 0x40080000. |
| SP = 0 | Still before our stack initialization | Check PC and execute through our ldr/mov instructions. |
| GDB says break x4 is invalid | Register contents are not symbol names | Use break *$x4. |
| x/c $x0 gives strange result for 0x49/0x54 | x0 currently contains a character value, not an address | Use p/c (char)$x0. |
| QEMU prints only TIMER TEST | IRQ entered but handler is not producing a usable INTID | Inspect ISPENDR0, IAR, HPPIR, then verify final GIC configuration. |
| GICC_IAR returns 0x3fe | Pending Group-1 interrupt is not being returned as INTID 27 through normal IAR | For this lab's QEMU GICv2 path, use GICC_CTLR=0x6 and normal IAR/EOIR. |
| GDB system-register expressions show void | Remote target does not expose those registers as ordinary GDB registers | Capture with MRS into volatile C variables and inspect the variables. |

# 12. GDB field cheat sheet used in the lab

| Question | Command |
| --- | --- |
| Where am I? | p/x $pc |
| What instruction is running? | x/10i $pc |
| Inspect registers | info registers |
| Inspect one register | info registers x0 |
| Hex register value | p/x $x0 |
| Character value | p/c (char)$x0 |
| Memory word | x/wx ADDRESS |
| String at pointer | x/s $x0 |
| Single instruction | si |
| Next C source line | next |
| Enter called function | step |
| Run until function return | finish |
| Break at symbol | break function_name |
| Break at exact address | break *0xADDRESS |
| Break at address stored in register | break *$x4 |
| Continue to next breakpoint | continue |
| Current line | info line |
| Call stack | bt |
| Breakpoints | info breakpoints |

# 13. One-pass reproduction of the current v0.5 checkpoint

This section is for a new engineer who wants the current completed result without rebuilding the history interactively. Use the full v0.5 source snapshot in Section 8.

Install the prerequisite packages in Section 3.

Create ~/project_Tiny_kernel/tiny-kernel with the exact tree shown in Section 3.

Create the v0.5 base source files exactly as shown in Section 8, then apply the v0.6 linker, Makefile, allocator, and test changes in Section 16.

Run make clean && make.

Run readelf entry/symbol checks before booting.

Run QEMU with -M virt,gic-version=2, -nographic, -monitor none, -kernel kernel.bin.

Confirm TIMER TEST followed by repeated TICK lines.

Optionally repeat the GDB validation in Sections 5, 6, 7, and 9.

## 13.1 Final file checklist

tiny-kernel/
├── Makefile
├── linker.ld
├── boot/
│   └── start.S
├── kernel/
│   ├── main.c
│   ├── uart.c
│   ├── exception.c
│   ├── exception.S
│   ├── gic.c
│   └── timer.c
├── drivers/
└── include/

# 14. Engineering lessons captured from the work

Never assume the first instruction at an address belongs to your source tree. Prove the running image with PC, disassembly, symbols, and the linker layout.

A register value is only meaningful in context. First identify whether it is a scalar value, pointer, return address, MMIO address, or boot argument.

MMIO registers are device state, not normal RAM. Writes and readbacks must be interpreted using the device model and access rules.

When an interrupt is missing, prove the chain from source -> timer -> pending bit -> CPU exception -> acknowledge -> handler -> EOI -> return instead of changing several subsystems at once.

A controlled failure is valuable. The BRK #0 exception in v0.4 and the forced GIC acknowledgement problem in v0.5 converted abstract architecture knowledge into observable bring-up evidence.

The project is intentionally a teaching kernel, not a production operating system. It does not yet implement full exception frames, scheduler, VM, locking, generic allocators, SMP, device tree, or Linux driver compatibility.

# 15. Current verified baseline and next entry point

CURRENT BASELINE: v0.6 PASS - QEMU virt + GICv2 + Cortex-A53 + raw kernel.bin + TIMER/TICK regression + minimal page-frame allocator + GDB bitmap proof.

v0.6 Minimal Memory Management is COMPLETE. The frozen v0.6 checkpoint is QEMU virt + GICv2 + Cortex-A53 + raw kernel.bin, with timer/TICK regression intact and the page-frame allocator validated by runtime tests and GDB.

## 15.1 v0.6 entry criteria

v0.5 output remains unchanged after each v0.6 build.

A 4 KiB page/frame allocation returns aligned addresses.

Allocated pages are tracked so double-allocation is detected by the test logic.

A free operation returns the page to the available pool.

GDB can verify the allocator state and returned addresses.

The result is recorded in the same document style before moving to v0.7.

# Appendix A - Final command reference

# Build
cd ~/project_Tiny_kernel/tiny-kernel
make clean
make

# ELF validation
aarch64-linux-gnu-readelf -h kernel.elf | grep "Entry point"
aarch64-linux-gnu-readelf -s kernel.elf | grep -E "_start|vector_table|kernel_main|irq_exception|stack_top"
aarch64-linux-gnu-objdump -d kernel.elf | less
od -Ax -tx1 -N32 kernel.bin

# Normal v0.5 run
qemu-system-aarch64 \
 -M virt,gic-version=2 \
 -cpu cortex-a53 \
 -m 128M \
 -nographic \
 -monitor none \
 -kernel kernel.bin

# Debug run
qemu-system-aarch64 \
 -M virt,gic-version=2 \
 -cpu cortex-a53 \
 -m 128M \
 -nographic \
 -monitor none \
 -kernel "$PWD/kernel.bin" \
 -S \
 -gdb tcp::1234

# GDB
gdb-multiarch ./kernel.elf
set architecture aarch64
target remote :1234

# Appendix B - "Do not repeat the old mistakes" list

Do not link the raw kernel with a build-id that lands before your startup code.

Do not assume 0x40000000 is your _start just because it is the RAM base shown by the platform.

Do not use set target :1234; use target remote :1234.

Do not use break x4; use break *$x4.

Do not confuse x/c $x0 with p/c (char)$x0.

Do not add -serial stdio together with -nographic for this known-good command.

Do not change timer code when the pending bit already proves the timer is firing.

Do not change several GIC registers at once without recording the evidence that motivated the change.

# Appendix C - Standalone release checklist

| Check | Required result | Pass |
| --- | --- | --- |
| Fresh machine prerequisites | gcc-aarch64-linux-gnu, binutils, qemu-system-aarch64, gdb-multiarch available | ☒ |
| Source completeness | v0.5 base + v0.6 allocator/linker/build/source snapshot + v0.7 one-task source are documented | ☒ |
| Build | make clean && make succeeds | ☒ |
| ELF | Entry point = 0x40080000 | ☒ |
| Boot | TIMER TEST appears | ☒ |
| Interrupt | Repeated TICK appears | ☒ |
| Debug reproducibility | GDB steps in Sections 5-9 work | ☒ |
| Regression discipline | v0.6-final is the frozen checkpoint before v0.7 | ☒ |
| v0.6 memory tests | PAGE1/PAGE2/PAGE3 aligned; FREE/REUSE PASS; NEGATIVE TEST PASS; EXHAUSTION PASS | ☒ |
| v0.6 GDB runtime proof | allocator_start=0x40093000; allocator_end=0x48000000; total=32621; bitmap=4078; initialized=1 | ☒ |
| v0.6 bitmap proof | bitmap changes 0x00 -> 0x01 and first allocation returns 0x40093000 | ☒ |
| v0.6 documentation | Main manual and v0.7 forensic log synchronized; next milestone not started | ☒ |

# 16. v0.6 - Minimal memory management (FINAL)

Goal: add a minimal 4 KiB page/frame allocator on the verified QEMU RAM range, track allocation state with a bitmap, support safe free/reuse, prove negative cases, stress exhaustion, and verify the live allocator state with GDB. v0.6 is complete.

## 16.1 Verified v0.6 memory map

ELF / linker symbols:
__bss_start = 0x40081030
__bss_end   = 0x40081068
stack_top   = 0x40091070
heap_start  = 0x40092000
heap_end    = 0x48000000

Raw heap range:
bytes  = 0x7F6E000
KiB    = 130488
4 KiB pages = 32622

Important distinction: linker heap_start (0x40092000) is the bitmap base. The allocator reserves 4078 bytes for the bitmap, aligns the usable frame area upward, and therefore begins page allocations at allocator_start = 0x40093000. The allocator manages 32621 usable 4 KiB frames.

## 16.2 Final linker addition

stack_top = .;
. = ALIGN(0x1000);
heap_start = .;
heap_end = ABSOLUTE(0x48000000);

## 16.3 Final allocator implementation

// kernel/page_alloc.c
#include <stdint.h>

#define PAGE_SIZE 0x1000UL
#define PAGE_MASK (PAGE_SIZE - 1)

extern char heap_start[];
extern char heap_end[];

volatile uintptr_t allocator_start;
volatile uintptr_t allocator_end;
volatile uint32_t allocator_total_pages;
volatile uint32_t allocator_bitmap_bytes;
volatile uint32_t allocator_initialized;

static uint8_t *page_bitmap;

static uintptr_t align_up(uintptr_t value)
{
    return (value + PAGE_MASK) & ~PAGE_MASK;
}

static void bitmap_clear(void)
{
    uint32_t i;
    for (i = 0; i < allocator_bitmap_bytes; i++)
        page_bitmap[i] = 0;
}

static int bitmap_test(uint32_t index)
{
    return (page_bitmap[index >> 3] &
            (uint8_t)(1U << (index & 7))) != 0;
}

static void bitmap_set(uint32_t index)
{
    page_bitmap[index >> 3] |=
        (uint8_t)(1U << (index & 7));
}

static void bitmap_clear_bit(uint32_t index)
{
    page_bitmap[index >> 3] &=
        (uint8_t)~(1U << (index & 7));
}

void page_alloc_init(void)
{
    uintptr_t start = (uintptr_t)heap_start;
    uintptr_t end = (uintptr_t)heap_end;
    uint32_t raw_pages =
        (uint32_t)((end - start) / PAGE_SIZE);

    allocator_bitmap_bytes =
        (raw_pages + 7U) / 8U;

    allocator_start =
        align_up(start + allocator_bitmap_bytes);

    allocator_end = end;

    allocator_total_pages =
        (uint32_t)((allocator_end - allocator_start)
                   / PAGE_SIZE);

    page_bitmap = (uint8_t *)start;
    bitmap_clear();
    allocator_initialized = 1;
}

void *page_alloc(void)
{
    uint32_t i;
    if (!allocator_initialized)
        return (void *)0;

    for (i = 0; i < allocator_total_pages; i++) {
        if (!bitmap_test(i)) {
            bitmap_set(i);
            return (void *)(allocator_start +
                            ((uintptr_t)i * PAGE_SIZE));
        }
    }
    return (void *)0;
}

int page_free(void *page)
{
    uintptr_t address;
    uint32_t index;

    if (!allocator_initialized || page == (void *)0)
        return -1;

    address = (uintptr_t)page;

    if (address < allocator_start ||
        address >= allocator_end)
        return -1;

    if ((address & PAGE_MASK) != 0)
        return -1;

    index =
        (uint32_t)((address - allocator_start) /
                   PAGE_SIZE);

    if (index >= allocator_total_pages)
        return -1;

    if (!bitmap_test(index))
        return -2;

    bitmap_clear_bit(index);
    return 0;
}

## 16.4 Build integration

The Makefile was extended with kernel/page_alloc.o and an explicit compile rule. main.c was connected to page_alloc_init(), page_alloc(), page_free(), and allocator_total_pages. uart_putc() was made public so the main memory tests could print hexadecimal values.

OBJS = boot/start.o \
       kernel/main.o \
       kernel/uart.o \
       kernel/exception.o \
       kernel/exception-vector.o \
       kernel/gic.o \
       kernel/timer.o \
       kernel/page_alloc.o

kernel/page_alloc.o: kernel/page_alloc.c
	$(CC) $(CFLAGS) -c $< -o $@

## 16.4A Final main.c memory acceptance sequence

page_alloc_init();

uart_puts("MEMORY TEST\r\n");

uart_puts("PAGE1 = 0x");
uart_puthex((uintptr_t)page_alloc());
uart_puts("\r\n");
uart_puts("PAGE2 = 0x");
uart_puthex((uintptr_t)page_alloc());
uart_puts("\r\n");
uart_puts("PAGE3 = 0x");
uart_puthex((uintptr_t)page_alloc());
uart_puts("\r\n");

void *free_test_page = page_alloc();
uart_puts("FREE TEST PAGE = 0x");
uart_puthex((uintptr_t)free_test_page);
uart_puts("\r\n");

int free_result = page_free(free_test_page);
uart_puts("FREE RESULT = ");
uart_puthex((uintptr_t)free_result);
uart_puts("\r\n");

void *reused_page = page_alloc();
uart_puts("REUSED PAGE = 0x");
uart_puthex((uintptr_t)reused_page);
uart_puts("\r\n");

if (reused_page == free_test_page)
    uart_puts("FREE/REUSE PASS\r\n");

page_free(reused_page);
int double_free_result = page_free(reused_page);
int invalid_free_result =
    page_free((void *)((uintptr_t)reused_page + 1));

uart_puts("DOUBLE FREE RESULT = 0x");
uart_puthex((uintptr_t)double_free_result);
uart_puts("\r\n");
uart_puts("INVALID FREE RESULT = 0x");
uart_puthex((uintptr_t)invalid_free_result);
uart_puts("\r\n");

if (double_free_result == -2 && invalid_free_result == -1)
    uart_puts("NEGATIVE TEST PASS\r\n");

uint32_t exhausted_count = 0;
void *exhausted_page;
while ((exhausted_page = page_alloc()) != (void *)0)
    exhausted_count++;

uart_puts("EXHAUST COUNT = 0x");
uart_puthex((uintptr_t)exhausted_count);
uart_puts("\r\n");
uart_puts("EXHAUST RESULT = 0x");
uart_puthex((uintptr_t)page_alloc());
uart_puts("\r\n");

if (exhausted_count == (allocator_total_pages - 3U) &&
    page_alloc() == (void *)0)
    uart_puts("EXHAUSTION PASS\r\n");

## 16.5 Final runtime acceptance output

PAGE1 = 0x0000000040093000
PAGE2 = 0x0000000040094000
PAGE3 = 0x0000000040095000

FREE TEST PAGE = 0x0000000040096000
FREE RESULT = 0000000000000000
REUSED PAGE = 0x0000000040096000
FREE/REUSE PASS

DOUBLE FREE RESULT = 0xfffffffffffffffe
INVALID FREE RESULT = 0xffffffffffffffff
NEGATIVE TEST PASS

EXHAUST COUNT = 0x0000000000007f6a
EXHAUST RESULT = 0x0000000000000000
EXHAUSTION PASS

TICK
TICK
TICK
...

Interpretation: 0x7f6a = 32618 additional allocations after the first three persistent pages. Together with PAGE1/PAGE2/PAGE3, that consumes all 32621 allocator-managed frames. The temporary FREE TEST PAGE was freed and successfully reused before exhaustion.

## 16.6 Final GDB runtime proof

(gdb) set architecture aarch64
(gdb) target remote :1234
(gdb) break page_alloc
(gdb) continue

(gdb) p/x allocator_start
$1 = 0x40093000
(gdb) p/x allocator_end
$2 = 0x48000000
(gdb) p allocator_total_pages
$3 = 32621
(gdb) p allocator_initialized
$4 = 1

(gdb) x/4bx 0x40092000
0x40092000:  0x00  0x00  0x00  0x00

(gdb) finish
Value returned is $5 = (void *) 0x40093000

(gdb) x/4bx 0x40092000
0x40092000:  0x01  0x00  0x00  0x00
(gdb) p/x $x0
$7 = 0x40093000

This is the runtime bitmap proof: before the first allocation, bit 0 is clear (0x00); page_alloc() returns 0x40093000; after the allocation, bit 0 is set (0x01).

## 16.7 v0.6 mistakes intentionally preserved

heap_end = 0x48000000 inside .bss produced 0x88081030; the fix was heap_end = ABSOLUTE(0x48000000).

The first sed alignment edit inserted a literal backslash before the dot: \. = ALIGN(0x1000);. The correction removed the backslash.

An early source insertion matched more than one location and caused duplicate declarations/redeclarations. The duplicated block was removed before rebuilding.

A cleanup edit removed too much of main.c and left the old asm/while tail. The stale lines were deleted and the test sequence rebuilt cleanly.

The exhaustion test appeared slow at -O0 because page_alloc() performs a linear scan on every allocation. It eventually completed; this was performance of the simple teaching allocator, not a deadlock or hang.

GDB command `remote target :1234` was invalid. The correct command is `target remote :1234`.

`finish` was incorrectly used while stopped in kernel_main(), which never returns. It was interrupted with Ctrl+C. `finish` is correct when stopped inside page_alloc(), because page_alloc() returns.

The typo `fisnish` was rejected by GDB; no kernel change was involved.

## 16.8 v0.6 completion gate

# 17. v0.7 - One-Task Execution

Goal: use the v0.6 page-frame allocator to allocate one dedicated 4 KiB task stack, switch SP to that stack, execute one task entry function, restore the kernel stack, mark the task DONE, free the task stack, and prove the complete path with QEMU and GDB.

## 17.1 v0.7 design boundary

This milestone deliberately implements one task only. It does not add a scheduler, timer-driven preemption, context switching between multiple tasks, locking, SMP, or user mode. The timer/GIC path remains the existing v0.6 regression path.

Kernel stack
    |
    | task_create()
    v
page_alloc() -> dedicated 4 KiB task stack
    |
    v
TASK_READY
    |
    | task_run()
    v
save kernel SP -> x19
set SP = task_stack_top
blr task_entry
    |
    v
TASK_DONE
    |
    v
restore kernel SP
    |
    v
TASK_DEAD after task_destroy()

## 17.2 New v0.7 files

// include/task.h
#ifndef TASK_H
#define TASK_H

#include <stdint.h>

#define TASK_READY    0
#define TASK_RUNNING  1
#define TASK_DONE     2
#define TASK_DEAD     3

typedef void (*task_entry_t)(void);

struct task {
    void *stack_page;
    uintptr_t stack_top;
    task_entry_t entry;
    int state;
};

int task_create(struct task *task, task_entry_t entry);
int task_run(struct task *task);
int task_destroy(struct task *task);

#endif

// kernel/task.c
#include "task.h"

#define PAGE_SIZE 0x1000UL

extern void *page_alloc(void);
extern int page_free(void *page);
extern void task_start(uintptr_t stack_top, task_entry_t entry);

int task_create(struct task *task, task_entry_t entry)
{
    void *page;

    if (task == (void *)0 || entry == (task_entry_t)0)
        return -1;

    page = page_alloc();
    if (page == (void *)0)
        return -2;

    task->stack_page = page;
    task->stack_top = (uintptr_t)page + PAGE_SIZE;
    task->entry = entry;
    task->state = TASK_READY;

    return 0;
}

int task_run(struct task *task)
{
    if (task == (void *)0 ||
        task->stack_page == (void *)0 ||
        task->entry == (task_entry_t)0 ||
        task->state != TASK_READY)
        return -1;

    task->state = TASK_RUNNING;
    task_start(task->stack_top, task->entry);
    task->state = TASK_DONE;
    return 0;
}

int task_destroy(struct task *task)
{
    int result;

    if (task == (void *)0 || task->stack_page == (void *)0)
        return -1;
    if (task->state == TASK_RUNNING)
        return -2;

    result = page_free(task->stack_page);
    if (result != 0)
        return result;

    task->stack_page = (void *)0;
    task->stack_top = 0;
    task->entry = (task_entry_t)0;
    task->state = TASK_DEAD;
    return 0;
}

// kernel/task_switch.S
.section .text
.global task_start
.type task_start, %function

task_start:
    stp x19, x20, [sp, #-16]!
    stp x29, x30, [sp, #-16]!
    mov x19, sp
    mov sp, x0
    blr x1
    mov sp, x19
    ldp x29, x30, [sp], #16
    ldp x19, x20, [sp], #16
    ret

## 17.3 Build integration

The Makefile gained -Iinclude and the two v0.7 objects kernel/task.o and kernel/task_switch.o, with explicit compile rules. The existing v0.6 objects remain unchanged.

CFLAGS += -Iinclude

OBJS += kernel/task.o kernel/task_switch.o

kernel/task.o: kernel/task.c include/task.h
	$(CC) $(CFLAGS) -c $< -o $@

kernel/task_switch.o: kernel/task_switch.S include/task.h
	$(CC) $(CFLAGS) $(ASFLAGS) -c $< -o $@

## 17.4 v0.7 acceptance tests in main.c

static volatile uint32_t demo_task_runs = 0;

static void demo_task(void)
{
    volatile uint64_t task_stack_marker = 0x1122334455667788ULL;

    demo_task_runs++;
    uart_puts("TASK START\r\n");

    if (task_stack_marker == 0x1122334455667788ULL)
        uart_puts("TASK RUNNING\r\n");
    else
        uart_puts("TASK STACK FAIL\r\n");

    uart_puts("TASK END\r\n");
}

After page_alloc_init():

TASK TEST

Prove:
- task_create(NULL, demo_task) -> -1
- task_create(&task, NULL) -> -1
- valid create -> 0
- state becomes TASK_READY
- stack_top = stack_page + 0x1000
- task_run() -> 0
- demo_task_runs == 1
- state becomes TASK_DONE
- task_destroy() -> 0
- state becomes TASK_DEAD
- stack_page cleared to NULL
- second task reuses the freed stack page
- task_run() on DEAD task -> -1
- task_destroy() on DEAD task -> -1
- demo_task_runs remains 1

## 17.5 Final v0.7 runtime output

TIMER TEST
TASK TEST
TASK CREATE PASS
TASK READY PASS
TASK START
TASK RUNNING
TASK END
TASK RUN PASS
TASK DESTROY PASS
TASK STACK REUSE PASS
TASK NEGATIVE PASS
TASK BODY PASS
MEMORY TEST
PAGE1 = 0x0000000040093000
PAGE2 = 0x0000000040094000
PAGE3 = 0x0000000040095000
FREE TEST PAGE = 0x0000000040096000
FREE RESULT = 0000000000000000
REUSED PAGE = 0x0000000040096000
FREE/REUSE PASS
DOUBLE FREE RESULT = 0xfffffffffffffffe
INVALID FREE RESULT = 0xffffffffffffffff
NEGATIVE TEST PASS
EXHAUST COUNT = 0x0000000000007f6a
EXHAUST RESULT = 0x0000000000000000
EXHAUSTION PASS
TICK
TICK
TICK
...

Result: PASS. The task subsystem executes successfully and the v0.6 allocator/timer regression remains intact after the v0.7 additions.

## 17.6 Final GDB stack-switch proof

(gdb) break task_start
(gdb) continue
Breakpoint 1, task_start () at kernel/task_switch.S:6

(gdb) info registers sp x0 x1
sp = 0x40091af0
x0 = 0x40094000
x1 = 0x40080090

(gdb) si
(gdb) si
(gdb) si

(gdb) info registers sp x19 x0
sp = 0x40091ad0
x19 = 0x40091ad0
x0 = 0x40094000

(gdb) break demo_task
(gdb) continue
Breakpoint 2, demo_task () at kernel/main.c:26

(gdb) info registers sp
sp = 0x40093fe0

(gdb) p/x &task_stack_marker
$1 = 0x40093ff8

(gdb) finish
Run till exit from #0 demo_task () at kernel/main.c:26
task_start () at kernel/task_switch.S:13
13 mov sp, x19

(gdb) si
(gdb) info registers sp x19
sp = 0x40091ad0
x19 = 0x40091ad0

Interpretation: x0 carries the dedicated task stack top 0x40094000. After mov sp, x0 the CPU executes demo_task on that stack. The local marker at 0x40093ff8 is inside the task page. After task return, mov sp, x19 restores the saved kernel stack at 0x40091ad0. The final ret returns to task_run().

## 17.7 v0.7 mistakes preserved

A scripted Makefile edit initially produced "missing separator" because the generated makefile recipe indentation was wrong. The Makefile was restored and rebuilt cleanly.

The first scripted main.c insertion did not match the actual v0.6 source layout. Rather than guessing, main.c was restored from v0.6-final and the v0.7 block was inserted against the known baseline.

The first task boot showed literal \r\n text because the generated C strings were double-escaped. The source was corrected to use real C escape sequences "\r\n" and the output became normal line-separated UART text.

A background QEMU/GDB attempt attached after execution had already advanced beyond task_start(); the breakpoint did not hit and task_stack_marker was not in the current kernel_main() context. The clean proof restarted QEMU paused with -S and set breakpoints before continue.

The GDB typo "remote target :1234" was invalid; the correct command is "target remote :1234".

finish is appropriate inside task functions that return, but not from kernel_main(), which intentionally waits forever in the WFE loop.

## 17.8 v0.7 completion gate

| Check | Required / observed result | Status |
| --- | --- | --- |
| Task create | Valid task gets a page, entry pointer, stack_top, and TASK_READY | PASS |
| Task execution | demo_task runs and increments its execution counter once | PASS |
| Task stack | SP switches to 0x40094000; local marker is in task page | PASS |
| Task return | SP restored to 0x40091ad0; execution returns to task_run() | PASS |
| Task destroy | Stack page freed and task transitions to TASK_DEAD | PASS |
| Stack reuse | Freed page is immediately reused by second task | PASS |
| Negative tests | Invalid create/run/destroy paths return expected errors | PASS |
| Regression | v0.6 allocator exhaustion and repeated TICK still pass | PASS |
| GDB | Instruction-level stack switch and restoration observed | PASS |
| Documentation | Main manual and dedicated v0.7 forensic log synchronized | PASS |

v0.7 is CLOSED. No next-milestone implementation should begin until this checkpoint is preserved in GitHub and the documentation is finalized.

Regression: v0.6 TIMER/TICK remains PASS after the v0.7 task tests.

Task lifecycle: create -> READY -> RUNNING -> DONE -> DEAD is PASS.

Task stack: dedicated 4 KiB page, stack switch, task-body local variable, and stack-page reuse are PASS.

GDB: task stack switch to 0x40094000 and kernel stack restoration to 0x40091ad0 are PASS.

Documentation: v0.7 main manual and dedicated forensic log are synchronized; the next milestone has not started.